# Urban Heat Island Intensity Analysis of Kolkata (2015–2025)

Shruti Chatterjee

Department of Geography (Independent Research Project)

# ABSTRACT

This study analyzes the temporal variation and intensity change of Urban Heat Islands (UHIs) in Kolkata over a decade, from 2015 to 2025, using satellite-derived Land Surface Temperature (LST) data. Landsat 8 (2015) and Landsat 9 (2025) thermal bands were processed in QGIS to derive LST and assess spatial patterns of surface temperature. A change detection analysis was conducted to  identify  areas of significant warming across the city, with temperature increases of up to approximately 6°C observed in the central and eastern urban sectors. Mean surface temperature rose by 2.57°C, and the area under high to very high UHI zones expanded by about 13.7%. These findings highlight the growing thermal stress associated with rapid urban expansion in Kolkata and underscore the need for sustainable planning measures such as vegetation enhancement, cool roofing, and wetland preservation to mitigate future heat effects.

Keywords: Urban Heat Island, Land Surface Temperature, Kolkata, Remote Sensing, QGIS, Sustainable Urban Planning

# 1. INTRODUCTION

Urban Heat Island (UHI) refers to the phenomenon where built-up city areas exhibit higher surface and air temperatures than their rural surroundings due to impervious surfaces, limited vegetation, and anthropogenic heat release. Kolkata, a major metropolitan city in eastern India, has undergone rapid urbanization in the last decade, resulting in significant alterations to its surface energy balance. Monitoring UHI intensity helps in understanding the relationship between urban growth and climate variability.

Previous studies across Indian cities such as Delhi, Mumbai, and Hyderabad have shown similar thermal intensification trends, yet few have examined Kolkata's decadal dynamics using consistent Landsat datasets. This study bridges that gap by analyzing temporal changes in LST between 2015 and 2025 and correlating them with urban expansion patterns.

# 2. STUDY AREA AND DATA

Kolkata, located along the eastern bank of the Hooghly River, lies between latitudes 22°31′N to 22°57′N and longitudes 88°17′E to 88°25′E. It is characterized by a tropical wet-and-dry climate with distinct pre-monsoon heat waves. Rapid population growth and industrialization have led to an increase in impervious surfaces. The study utilizes Landsat 8 and 9 imagery for the years 2015 and 2025 respectively, both sourced from the USGS  Earth Explorer portal.

# 3. METHODOLOGY

The study followed a remote sensing and GIS-based workflow including data acquisition, preprocessing, LST derivation, UHI classification, and change detection. Radiometric and atmospheric corrections were applied, followed by conversion of digital numbers (DN) to top-of-atmosphere radiance and brightness temperature. Land Surface Temperature (LST) was calculated using emissivity-corrected equations within QGIS 3.40. The resulting maps were classified into five UHI intensity levels

.

Workflow Diagram for UHI Analysis

# 4. RESULTS AND DISCUSSION

The UHI map of 2015 reveals a spatial LST variation between 28.38°C and 46.75°C, while the 2025 map shows a range between 28.56°C and 50.52°C. High-temperature zones are concentrated in the core and eastern sectors of the city. Change detection indicates an average surface temperature increase of 2.57°C, with substantial expansion in high-intensity heat zones.

| Year | Satellite | Sensor | Path/Row | Acquisition Date | Spatial Resolution (m) | Thermal Band Used |
| --- | --- | --- | --- | --- | --- | --- |
| 2015 | Landsat 8 | TIRS | 138/44 | May 2015 | 30 | Band 10 |
| 2025 | Landsat 9 | TIRS-2 | 138/44 | May 2025 | 30 | Band 10 |

Table . . Satellite Data Overview

| Year | Minimum LST (°C) | Maximum LST (°C) | Mean LST (°C) | Standard Deviation (°C) |
| --- | --- | --- | --- | --- |
| 2015 | 28.38 | 46.75 | 37.56 | 4.22 |
| 2025 | 28.04 | 50.52 | 40.13 | 5.06 |

Table : Showing land surface temperature (2015 and2025)

| UHI Category | Temperature Range (°C) | 2015 Area (%) | 2025 Area (%) | Change (Δ%) |
| --- | --- | --- | --- | --- |
| Very Low | <30 | 22.4 | 15.3 | -7.1 |
| Low | 30–34 | 28.7 | 22.5 | -6.2 |
| Moderate | 34–38 | 25.1 | 24.7 | -0.4 |
| High | 38–42 | 17.8 | 24.9 | +7.1 |
| Very High | >42 | 6.0 | 12.6 | +6.6 |

Table . Showing UHI category

| Zone Type | Key Locations | Observed Trend (2015–2025) | Likely Cause |
| --- | --- | --- | --- |
| Core Urban | Burrabazar, Esplanade, South Kolkata | Significant rise (up to +6°C) | Dense built-up area, high vehicular emission |
| Eastern Sector | Salt Lake, Rajarhat | Moderate rise | New residential and IT infrastructure |
| Southern Fringe | Jadavpur, Garia | Slight increase | Expanding residential zones |
| River-adjacent | Along Hooghly River | Stable / cooling | Presence of water and green buffers |

Table . Showing zonal characteristics

NDVI–LST Correlation Analysis

To statistically evaluate the relationship between vegetation cover and surface temperature, Pearson’s correlation analysis was conducted between NDVI and Land Surface Temperature (LST) for 2015 and 2025.

The results indicate a moderate negative correlation in both years:

- 2015: r = -0.358

- 2025: r = -0.338

The negative correlation confirms that areas with higher vegetation density experienced lower surface temperatures, demonstrating the cooling effect of urban greenery.

However, a slight reduction in correlation strength in 2025 suggests increasing urban surface heterogeneity and possible fragmentation of vegetated areas. This indicates that while vegetation continues to mitigate thermal intensity, its effectiveness may be diminishing due to expanding impervious surfaces.

These findings statistically validate the observed spatial patterns of Urban Heat Island expansion across Kolkata

| Year | Correlation Coefficient (r) | Relationship Strength | Interpretation |
| --- | --- | --- | --- |
| 2015 | -0.358 | Moderate Negative | Vegetation reduces LST |
| 2025 | -0.338 | Moderate Negative | Cooling effect slightly weakened |

Spatial patterns demonstrate that central commercial and residential regions experienced the greatest temperature increase, attributed to high impervious surface density and limited vegetation. Conversely, river-adjacent and wetland zones exhibited cooling trends, reflecting the influence of hydrological and vegetative features.

| Parameter | 2015 | 2025 | Decadal Change |
| --- | --- | --- | --- |
| Mean Surface Temp (°C) | 37.56 | 40.13 | +2.57 |
| Maximum Surface Temp (°C) | 46.75 | 50.52 | +3.77 |
| Area under “High” & “Very High” UHI (%) | 23.8 | 37.5 | +13.7 |

.

Table  : Decadal Urban Thermal Risk Indicators (2015–2025)

The results indicate a substantial intensification of surface thermal conditions across Kolkata between 2015 and 2025. The mean surface temperature increased by 2.57°C, suggesting an overall upward shift in the city’s thermal baseline. More critically, the maximum surface temperature rose by 3.77°C, indicating growing exposure to extreme heat conditions.

The most significant transformation is the 13.7% expansion in areas classified as High and Very High UHI zones. This expansion reflects spatial amplification of thermal hotspots, likely associated with accelerated built-up growth and vegetation decline.

Such escalation has implications for public health vulnerability, energy demand, and infrastructure stress in high-density urban sectors.

# 5. CONCLUSION AND POLICY IMPLICATIONS

The comparative analysis confirms a notable rise in UHI intensity across Kolkata between 2015 and 2025. The expansion of high-temperature zones correlates with unregulated urban growth and vegetation loss. Key mitigation measures include expanding green belts, implementing cool roofing systems, and protecting wetlands.

The study aligns with the objectives of Sustainable Development Goal (SDG) 11 – Sustainable Cities and Communities, and SDG 13 – Climate Action. Future research can integrate socio-economic parameters and field-based temperature validation to enhance model accuracy.

- REFERENCES

Oke, T. R. (1982). The energetic basis of the urban heat island. Quarterly Journal of the Royal Meteorological Society, 108(455), 1–24. https://doi.org/10.1002/qj.49710845502

Voogt, J. A., & Oke, T. R. (2003). Thermal remote sensing of urban climates. Remote Sensing of Environment, 86(3), 370–384.  https://doi.org/10.1016/S0034-4257(03)00079-8

Rasul, G., Balakrishnan, P., & Ahmed, M. (2017). Monitoring of urban heat island effect in Delhi using Landsat data. Environmental Monitoring and Assessment, 189(4), 161. https://doi.org/10.1007/s10661-017-5844-5

U.S. Geological Survey. (2023). Landsat Surface Temperature Science Product (Collection 2) [Data set]. https://www.usgs.gov/landsat-missions

Singh, R. B., & Grover, A. (2015). Urban heat island effect analysis using Landsat data: A case study of Delhi. Geography Journal, 2015, 1–12. https://doi.org/10.1155/2015/492619

Chakraborty, S., & Maitra, A. (2020). Impact of urbanization on surface temperature in eastern India: A remote sensing approach. Urban Climate, 34, 100707. https://doi.org/10.1016/j.uclim.2020.100707

United Nations Development Programme. (2022). Sustainable Development Goals: Goal 11—Sustainable cities and communities; Goal 13—Climate action. https://www.undp.org/sustainable-development-goals

QGIS Development Team. (2024). QGIS Geographic Information System (Version 3.x). Open Source Geospatial Foundation. https://qgis.org

NASA & U.S. Geological Survey. (2024). Landsat-8 Operational Land Imager (OLI) and Thermal Infrared Sensor (TIRS) data [Data set]. https://earthexplorer.usgs.gov

NASA & U.S. Geological Survey. (2024). Landsat-9 Operational Land Imager-2 (OLI-2) and Thermal Infrared Sensor-2 (TIRS-2) data [Data set]. https://earthexplorer.usgs.gov